Project Name:

SECTION 12492

Horizontal Louver Blinds

# Part 1 - General

# DESCRIPTION

- 2" Horizontal Louver Blinds With Aluminum Slats.

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Submit working hand sample or mock up blind as required.

- Submit two 6" samples of aluminum slat indicating color and dimensions.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions LLC or architect approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - HORIZONTAL BLINDS

- Product: Bali® Heritage™ 2” Aluminum Blinds

- Color Name:       Color Number:

- SureClose® Headrail shall be 1 5/8" high x 2 1/4" wide x .022” thick U-shaped steel with 1/8” light blocking lip on the bottom centerline. The steel finishing process includes phosphate treatment for corrosion resistance, a chrome-free sealer, a low HAP urethane primer and a topcoat with low HAP polyester baked enamel.

- Cord tilter shall be a snap-in component incorporating a worm and pulley of low-friction thermoplastic and a nylon gear.

Select One:

Cord tilter (standard 1.8mm in diameter)

Wand tilter

Ring tilter

- Cord lock shall be metal of a snap-in design incorporating a floating, shaft-type locking pin and shall incorporate a crash proof safety feature that will lock blind automatically upon release of cord.

Options:

Ring pull provides a single plated steel ring in lieu of    tassels with a nominal 4" cord length.

- Braided ladder shall be made of 100% polyester yarn incorporating two extra strength rungs per ladder for slat support.

Select One:

Braided Ladder (standard) 44mm

Fabric Tape

Color Name:       Color Number:

- Slats shall be 5000 series cold-rolled aluminum containing the maximum allowable recycled content to produce a high strength and corrosion resistant flexible product. Slats shall be nominally 2” wide x .008" thick and processed with Advanced Finishing Technology (AFT), providing a smooth, hard, less porous surface.  AFT delivers anti-static performance to repel dust. Slats shall be treated with a topcoat of polyester baked enamel.

- Bottomrail shall be “C” shaped 9/16” high x 2” wide x .040 thick anodized aluminum. It is fully enclosed with a dust cover slat and finished with a polyester baked enamel to match headrail.

  - Fabrication

- Blinds shall be fabricated according to specifications and accurate to tolerance established by SWF engineering standards

# Part 3 – Execution

  - Inspection

- installer shall be responsible for inspection of jobsite, approval of mounting surfaces, verification of field measurements and installation conditions.  Installation shall commence when satisfactory conditions are met.

  - Installation

- Install blinds in accordance with manufacturer’s instructions including recommended support brackets and fasteners.

- Install blinds with adequate clearance to permit smooth operation of the blinds.  Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

- Maintain and clean blinds in accordance with manufacturer’s instructions.

### End of Section