Bead Chain Clutch Operated FlexShades

** NOTE TO SPECIFIER ** Draper, Inc.; window shade products.
 .
 This section is based on the products of Draper, Inc., which is located at:
411 S. Pearl P. O. Box 425 
Spiceland, IN 47385-0425
Toll Free Tel: 800-238-7999
Tel: 765-987-7999
Fax: 866-637-5611
Email: drapercontract@draperinc.com
Web: www.draperinc.com
 
 Draper is a leader in the manufacture of window shades, projection screens, and gymnasium equipment. Founded in 1902, Draper maintains a tradition of quality and service. Draper products include FlexShade roller-operated and fixed window shades.
 
 Window shades reflect light and heat from the sun, reducing the amount of energy required to cool the building. Motorized window shades can automatically adjust to compensate for the changing shade requirements throughout the day. DRAPER's solar control shading systems can contribute to LEED points and credits. In addition, shades can reduce or eliminate uncomfortable glare for workers, while still allowing ambient sunlighting and view-through to the outside. The construction of window shades has become more friendly to the environment as well by including PVC-free fabrics, recyclable fabrics and lean manufacturing processes.
 
 Our spectrum of fabric colors and styles is unmatched anywhere. And we offer the broadest variety of operator styles in the industry.
 
 This section can be used for specifying Draper Chain and Clutch Operated FlexShade.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER **  Delete items below not required for project.

    - Manually operated, roll-up fabric interior window shades including mounting and operating hardware.

  - RELATED SECTIONS

** NOTE TO SPECIFIER **  Delete any sections below not relevant to this project; add others as required.

    - Section 06 10 00 - Rough Carpentry.

    - Section 07 90 00 - Joint Protection.

    - Section 09 21 16 - Gypsum Board Shaft Wall Assemblies.

    - Section 09 51 23 - Acoustical Tile Ceilings.

    - Section 11 52 13 - Projection Screens.

    - Section 11 52 16 - Projectors.

  - REFERENCES

** NOTE TO SPECIFIER **  Delete references from the list below that are not actually required by the text of the edited section.

    - NFPA 701-99 - Fire Tests for Flame-Resistant Textiles and Films.

    - GREENGUARD Environmental Institute Children & Schools

    - US Green Building Council.

  - SUBMITTALS

    - Submit under provisions of Section 01 33 26 - Source Quality Control Reporting:

    - Product Data:  Manufacturer's data sheets on each product specified, including:

      - Preparation instructions and recommendations.

      - Installation and maintenance instructions.

      - Styles, material descriptions, dimensions of individual components, profiles, features, finishes and operating instructions.

      - Storage and handling requirements and recommendations.

      - Mounting details and installation methods.

** NOTE TO SPECIFIER **  Include the following paragraph for projects with complex requirements for window treatment.  Delete if not required.

    - Shop Drawings:  Plans, elevations, sections, product details, installation details, operational clearances, wiring diagrams and relationship to adjacent work.

** NOTE TO SPECIFIER **  Delete the following paragraphs if LEED is not applicable.

    - LEED Submittals:  Provide documentation of how the requirements of Credit will be met.

    - Window Treatment Schedule:  For all roller shades.  Use same room designations as indicated on the Drawings, field verified window dimensions, quantities, type of shade, controls, fabric, and color, and include opening sizes and key to typical mounting details.

** NOTE TO SPECIFIER **  Delete Selection Samples paragraph if colors and shade fabrics have already been selected.

    - Selection Samples:  For each finish product specified, two complete sets of shade cloth options and aluminum finish color samples representing manufacturer's full range of available colors and patterns.

    - Verification Samples:  For each finish product specified, two complete sets of shade components, unassembled, demonstrating compliance with specified requirements.  Shade fabric sample and aluminum finish sample as selected, representing actual product, color, and patterns.  Mark face of material to indicate interior faces.

    - Maintenance Data:  Methods for maintaining roller shades, precautions regarding cleaning materials and methods, instructions for operating hardware and controls.

    - Manufacturer's Certificates: Certify products meet or exceed specified requirements.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Obtain roller shades through one source from a single manufacturer with a minimum of twenty years experience in manufacturing products comparable to those specified in this section.

    - NFPA Flame-Test: Passes NFPA 701. Materials tested shall be identical to products proposed for use.

** NOTE TO SPECIFIER **  Include a mock-up if the project size and/or quality warrant taking such a precaution.  The following is one example of how a mock-up on a large project might be specified.  When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up:  Provide a mock-up of one of each type roller shade assembly specified for evaluation of mounting, appearance and accessories.

      - Locate mock-up in window(s) designated by Architect.

      - Do not proceed with remaining work until mock-up is accepted by Architect.

  - DELIVERY, STORAGE, AND HANDLING

    - Do not deliver window shades until building is enclosed and construction within spaces where shades will be installed is substantially complete.

    - Deliver products in manufacturer's original, unopened, undamaged containers with labels intact.

    - Label containers and shades according to Window Shade Schedule.

    - Store products in manufacturer's unopened packaging until ready for installation.

  - SEQUENCING

    - Ensure that locating templates and other information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

  - PROJECT CONDITIONS

    - Install roller shades after finish work and ambient temperature, humidity and ventilation conditions are maintained at levels recommended for project upon completion.

  - WARRANTY

    - Hardware and Shade Fabric:  Draper's standard twenty-five year limited warranty.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Draper, Inc., which is located at: 411 S. Pearl P. O. Box 425 ; Spiceland, IN 47385-0425; Toll Free Tel: 800-238-7999; Tel: 765-987-7999; Fax: 866-637-5611; Email:drapercontract@draperinc.com; Web:www.draperinc.com

** NOTE TO SPECIFIER **  Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions:  Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00.

** NOTE TO SPECIFIER **  Select the shades required from the following paragraphs. For each Draper window shade there are options for size, fabric, operator, operator location, controls, method of installation, hardware, and hardware finish. Refer to Draper product literature for availability of options and selection guidelines. Various options are listed in this specification section, but not all options apply to all window shades. The specifier will need to select the appropriate options, verify their availability, and delete non-applicable models and options.

  - BEAD CHAIN CLUTCH OPERATED WINDOW SHADES

    - Manually Operated Window Shades with Independent Control: Manually operated, vertical roll-up, fabric window shade with components necessary for complete installation; Manual FlexShade as manufactured by Draper, Inc.

      - Operation: Bead chain and clutch operating mechanism allowing shade to stop when chain is released. Designed never to need adjustment or lubrication. Provide limit stops to prevent shade from being raised or lowered too far.

        - Clutch mechanism: Fabricated from high carbon steel and molded fiberglass reinforced polyester or injected molded nylon.

** NOTE TO SPECIFIER **  Select stainless steel (standard) or optional plastic bead chain from the following two paragraphs and delete the one not required.

        - Bead chain loop: Stainless steel bead chain hanging at side of window.

        - Bead chain loop: Plastic bead chain hanging at side of window, Ivory, Grey, or Black color as selected by Architect.

        - Idler Assembly: Provide roller idler assembly of molded nylon with adjustable length idler pin to facilitate easy installation, and removal of shade for service.

** NOTE TO SPECIFIER ** Select the mounting type required for window shade from the following options, delete those not applicable. If multiple types are required indicate the location of each in the schedule at the end of this section.

      - Mounting:

        - Mounting brackets.

        - Endcaps only.

        - Endcaps and fascia.

        - Endcaps and headbox.

        - Ceiling pocket.

      - Roller Tube: Fabricated from extruded aluminum, galvanized steel, or enameled steel. Diameter, wall thickness, and material selected by manufacturer to accommodate shade type and size. Fabric connected to the roller tube with LSE (low surface energy) double sided adhesive specifically developed to attach coated textiles to metal. Adhesive attachment to eliminate horizontal impressions in fabric.

** NOTE TO SPECIFIER **  Include endcaps if required and delete if not required. Endcaps are required for attachment of fascia and headbox.

      - Endcaps: Stamped steel with universal design suitable for mounting to ceiling, wall, and jamb. Provide size compatible with roller size.

        - Endcap covers: To match fascia or headbox color.

** NOTE TO SPECIFIER **  Shades provided without endcaps will require mounting brackets. Delete if not required.

      - Brackets: Plated stamped steel. Provide size compatible with roller size.

** NOTE TO SPECIFIER **  Select bracket mounting type required from the following and delete those not applicable. If multiple types are required indicate the location of each in the schedule at the end of this section.

        - Mounted to ceiling.

        - Mounted to wall.

        - Mounted to jamb.

** NOTE TO SPECIFIER **  Draper manually operated shades can be provided with aluminum fascia concealing shade roller and hardware. Fascia can be mounted inside window opening extending from jamb to jamb or mounted outside and overlapping window opening. Fascia is attached to endcaps. Fascia can be provided in lengths up to 16 feet and either 3-7/8 or 4-5/8 inches high. Fascia available for reverse roll applications. Include the following paragraph if fascia is required. Delete if not required.

      - Fascia: L shaped aluminum extrusion to conceal shade roller and hardware.

        - Attachment: Snaps onto endcaps without requiring exposed fasteners of any kind. Fascia can be mounted continuously across two or more shade bands.

** NOTE TO SPECIFIER **  Select the finish required from the following paragraphs and delete those not required.

        - Finish: Clear anodized.

        - Finish: Black powder coat.

        - Finish: White powder coat.

        - Finish: Ivory powder coat.

        - Finish: Bronze powder coat.

        - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER **  Draper manually operated shades can be provided with aluminum headbox designed for exposed surface or recessed ceiling installation. Ceiling/Wall style headbox consists of removable closure, back and top, and endcaps. Pocket style headbox consists of removable closure, U-Shaped pocket and endcaps. Headbox is provided in two sizes. Refer to Draper product literature for required sizes based on window and roller size. Include one of the following paragraphs if headbox is required. Delete if not required.

      - Headbox Ceiling/Wall style: Aluminum fabrication with removable closure, endcaps, and back and top cover piece:

** NOTE TO SPECIFIER **  Select the finish required from the following paragraphs and delete those not required.

        - Finish: Clear anodized.

        - Finish: Black powder coat.

        - Finish: White powder coat.

        - Finish: Ivory powder coat.

        - Finish: Bronze powder coat.

        - Finish: Custom powder coat as selected by the Architect.

      - Headbox, Pocket Style: Aluminum fabrication with removable closure, endcaps, and U-shaped pocket:

** NOTE TO SPECIFIER **  Select the finish required from the following paragraphs and delete those not required.

        - Finish: Clear anodized.

        - Finish: Black powder coat.

        - Finish: White powder coat.

        - Finish: Ivory powder coat.

        - Finish: Bronze powder coat.

        - Finish: Custom powder coat as selected by the Architect.

** NOTE TO SPECIFIER **  Select one of the following two paragraphs for Shade Pockets or Wall Clip with Closure Panel as required and delete the paragraph not required.

** NOTE TO SPECIFIER **  Chain and Clutch Operated FlexShades can be provided with and installed in fabricated pockets. Type D Pockets are designed to be installed prior to or as part of ceiling system installation. Window shade assembly can be site installed later after construction operations that might damage shade are complete. Include the following paragraph if Type D Pockets are required. Delete if not required.

      - Type D Shade pocket: Rectangular pocket designed for recessed ceiling installation of window shades.

        - Material: Extruded aluminum alloy with white epoxy paint finish.

        - Size: 4-1/2 inches (114 mm) wide by 4-5/8 inches (117 mm) high.

** NOTE TO SPECIFIER **  Closure panel is optional select if required or delete if not required.

        - Closure panel: Removable closure panel clips to bottom of pocket to form 1 inch (25 mm) opening for passage of shade fabric.

        - Provide pocket with 7/8 inch (22 mm) lip at lower edge for support of acoustical ceiling panel.

** NOTE TO SPECIFIER **  Endcaps for Type D FlexShade Pocket are provided with acoustical ceiling panel support lip.

        - Provide endcaps with 7/8 inch (22 mm) lip for support of acoustical ceiling panel.

** NOTE TO SPECIFIER **  Type D FlexShade Pocket can be provided with corner sections where window shade recess continues around two walls. Pocket ends can also be provided. Choose all that apply.

        - Corners: Welded one-piece aluminum sections connecting to and matching pockets to allow continuous shade recess at ceiling corners.

        - Pocket ends: Welded one-piece aluminum sections connecting to and matching pockets.

** NOTE TO SPECIFIER **  Draper manually operated window shades can be installed in a ceiling recess site constructed from light gage metal framing and gypsum board or other materials. For this condition, use the following paragraph to specify aluminum removable closure panel to be installed leaving a minimal slot for passage of shade. Delete if not required.

      - Wall Clip with Closure panel: For site constructed ceiling recesses, provide removable closure panel to minimize slot for shade passage but allowing access to shade for maintenance.

        - Material: Aluminum alloy with white epoxy paint finish.

** NOTE TO SPECIFIER **  Select closure width required from the following paragraphs.

        - Closure width: 1-3/4 inches (44 mm).

        - Closure width: 3 inches (75 mm).

        - Provide continuous wall clip, 1-13/16 (45 mm) by 1/4 inch (6 mm), for snap-in attachment of closure panel without fasteners.

  - FABRIC

** NOTE TO SPECIFIER ** Refer to Draper product literature for available shade fabrics and selection guidelines. Select fabric(s), percent open and color as applicable from the following and delete those not applicable. Use the schedule at the end of this section if required to define location(s) of multiple fabrics.

    - Light-Filtering Fabrics

      - SheerWeave Infinity by Phifer: Composed of 30 percent ThermoPlastic Oletin (TPO) fiberyarn and 70 percent TPO coating on TPO yarn. Made of 100 percent recycled post-industrial products. Recyclable, lead free, PVC free. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA-701-1999TM#1 (small scale). Microbial and fungal resistant.

** NOTE TO SPECIFIER ** Select percent open as applicable from the following and delete those not applicable.

        - Average 3 percent open. Available in 63 inch, 84 inch and 98 inch roll widths.

        - Average 5 percent open. Available in 63 inch, 84 inch and 98 inch roll widths.

      - SheerWeave Series SW3000 by Phifer: Vinyl coated fiberglass and vinyl coated polyester woven into 62 by 20 mesh. Fire rating: NFPA 701. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. 13.46 oz/sq yd, .028 inches thick. Average 14 percent open.

      - SheerWeave Series SW2000 by Phifer: VOC Emissions: GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: NFPA 701-1999TM#1 (small scale), NFPA 101 (Class A Rating), UBC (Class 1).. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. 14.26 oz/sq yd, .019 inches thick. Series SW2000 average 5 percent open.

      - SheerWeave Series SW2100 by Phifer: VOC Emissions: GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: NFPA 701-1999TM#1 (small scale), NFPA 101 (Class A Rating), UBC (Class 1).. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. 14.26 oz/sq yd, .019 inches thick. Series SW2100 average 10 percent open.

      - SheerWeave Series SW2500 by Phifer: VOC Emissions: GREENGUARD Children & Schools -certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: NFPA 701. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series SW2500, 1 percent open, .024 inches thick.

      - SheerWeave Series SW2400 by Phifer: VOC Emissions: GREENGUARD Children & Schools -certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: NFPA 701. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series SW2400, 3 percent open, .019 inches thick.

      - SheerWeave Series SW2900 by Phifer: VOC Emissions: GREENGUARD Children & Schools -certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: NFPA 701. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series SW2900, 5 percent open, .017 inches thick.

      - SheerWeave Series SW2600 by Phifer: VOC Emissions: GREENGUARD Children & Schools -certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: NFPA 701. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series SW2600, 10 percent open, .017 inches thick.

      - Series SW2701 SheerWeave: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701. SW2701—.1 percent open.

      - Series SW2703 SheerWeave: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701. SW2703—3 percent open.

      - Series SW2705 SheerWeave: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701. SW2705—5 percent open.

      - Series SW2710 SheerWeave: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701. SW2710—10 percent open.

      - SheerWeave Series SW5000 by Phifer: Polyester and vinyl coated polyester woven into a 42 by 20 mesh. Fire rating: NFPA 701 TM#1/California Technical Bulletin 117 Sect. E. Part 1/NFPA 101 (Class A Rating)/UBC (Class 1)/British Standard 5867. Bacteria and Fungal Resistance: ASTM-G21 and ASTM G 22. 12.3 to14.5 oz/sq yd, .035 inches - .044 inches thick, depending on color. 5 to 10 percent open, depending on color.

      - SheerWeave Series PW4400 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: NFPA 701 TM#1(small scale)/California U.S. title 19 (small scale)/NFPA 101 (Class A Rating)/UBC (Class 1)/British Standard 5867/NFPA 701-TM#2 large scale/CAN/ULC S109-M-87. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series PW4400 Average 3 percent open, 20.7 oz/sq yd, .037 inches thick.

      - SheerWeave Series PW3500 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: NFPA 701 TM#1(small scale)/California U.S. title 19 (small scale)/NFPA 101 (Class A Rating)/UBC (Class 1)/British Standard 5867/NFPA 701-TM#2 large scale/CAN/ULC S109-M-87. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series PW3500 Average 5 percent open, 19.2 oz/sq yd, .036 inches thick.

      - SheerWeave Series PW4100 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: NFPA 701 TM#1(small scale)/California U.S. title 19 (small scale)/NFPA 101 (Class A Rating)/UBC (Class 1)/British Standard 5867/NFPA 701-TM#2 large scale/CAN/ULC S109-M-87. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series PW4100 Average 10 percent open, 17.5 oz/sq yd, .035 inches thick.

      - SheerWeave Series PW4600 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: NFPA 701 TM#1(small scale)/California U.S. title 19 (small scale)/British Standard 5867 Type B/ASTM E 84 (Class 1). Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series PW4600, Average 3 percent open, 17.4 oz/sq yd, .030 inches thick.

      - SheerWeave Series PW4500 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: NFPA 701 TM#1(small scale)/California U.S. title 19 (small scale)/British Standard 5867 Type B/ASTM E 84 (Class 1). Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series PW4500, Average 5 percent open, 14.4 oz/sq yd, .024 inches thick.

      - SheerWeave Series PW4650 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: NFPA 701 TM#1(small scale)/California U.S. title 19 (small scale)/British Standard 5867 Type B/ASTM E 84 (Class 1). Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series PW4650, Average. 3 percent open, 13.3 oz/sq yd, .026 inches thick.

      - SheerWeave Series PW4550 by Phifer: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: NFPA 701 TM#1(small scale)/California U.S. title 19 (small scale)/British Standard 5867 Type B/ASTM E 84 (Class 1). Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. Series PW4550. Average 5 percent open, 12.5 oz/sq yd, .025 inches thick.

      - SheerWeave PW4800 by Phifer: Vinyl coated polyester with maximum UV blockage. Fire rating: NFPA 701-1999 TM#1(small scale)/California U.S. title 19 (small scale)/NFPA 101 (Class A Rating)/UBC (Class 1)/British Standard 5867. Bacteria and Fungal Resistance: ASTM G 21 and ASTM G 22. 1 percent openness factor. 18.5 oz/sq yd, .030 inches thick.

      - E Screen ME-01 Series by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701, both small- and large-scale tests/California U.S. Title 19. ME-01 Series: Avg. 1 percent open.

      - E Screen ME-03 Series by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701, both small- and large-scale tests/California U.S. Title 19. ME-03 Series: Avg. 3 percent open.

      - E Screen ME-05 Series by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701, both small- and large-scale tests/California U.S. Title 19. ME-05 Series: Avg. 5 percent open.

      - E Screen ME-10 Series by Mermet: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701, both small- and large-scale tests/California U.S. Title 19. ME Series: ME-10 Series: Avg. 10 percent open, 10.3 oz/sq yd. Okotex-labeled.

      - M Screen by Mermet: PVC coated fiberglass in 1 by 2 weave. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701, both small- and large-scale tests/California U.S. Title 19. Okotex-labeled.

** NOTE TO SPECIFIER ** Select percent open as applicable from the following and delete those not applicable.

        - 3 percent openness 12.7 oz/sq yd, .022 inches thick.

        - 5 percent openness 11.9 oz/sq yd, .022 inches thick.

      - T Screen 9601 by Mermet: PVC coated fiberglass woven mesh. Dark on one side, light on the other. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701-99 TM #1, California U.S. Title 19. 12.1 oz/sq yd, .028 inches thick. T Screen 9601: Avg. 1 percent open. Okotex-labeled.

      - T Screen 9603 by Mermet: PVC coated fiberglass woven mesh. Dark on one side, light on the other. GREENGUARD Children & Schools certified as a low emitting fabric. Manufacturer to supply GREENGUARD Children & Schools certificate. Fire rating: NFPA 701-99 TM #1, California U.S. Title 19. 12.1 oz/sq yd, .028 inches thick. T Screen 9603: Average 3 percent open.

      - SilverScreen by Mermet: PVC coated fiberglass. Approximately 4 percent open. 12.86 oz./ sq. yd., .020 inch thick. Fire rating: NFPA 701, M1, ISO 6941 and BS 5867 Part 2 Type B. Easily accepts graphics. Fabric has a metallic silver backing on the outside.

      - Soltis 99 by Ferrari: PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. 3 percent openness factor. 9 oz/sq yd, .0126 inches thick. Fire rating: NFPA 701. Easily accepts graphics. Recyclable, Okotex-labeled.

      - Linen Veil by Newcastle: 30 percent modified acrylic, 5 percent linen with PVC coated fiberglass yarns in an otherwise traditional 2 by 2 basket weave. 10 percent open. 13.7 oz/sq yd, .03 inches thick. Fire rating: NFPA 701.

      - Terra by Mermet: PVC-free fabric made from 100 percent FR polyester. 6 oz./sq. yd., .019 inches thick. 100 percent recyclable. Fire rating: NFPA 701, small-scale test.

      - Vela by Mermet: PVC-free fabric made from 100 percent FR polyester. Approximately 3 percent open. 6 oz./sq. yd., .015 inches thick. 100 percent recyclable. Fire rating: NFPA 701, small-scale test.

    - Room Darkening Fabrics

      - SunBloc Series SB9000: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1. Washable and stain resistant. Wt. 12 oz/sq yd. Same color both sides, .015 inches thick.

      - SunBloc Series SB9100: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1. Washable and stain resistant. Wt. 12 oz/sq yd. White exterior in all colors, .013 inches thick.

      - CL8000 (Colony): Close woven fiberglass base textile with sun-resistant vinyl film securely bonded to each side. Fire rating: NFPA 701 Test 1996-1. Washable and stain resistant. 90 percent opaque. 9.0 oz/sq yd., .014 inches thick. Fabric is white on outside.

      - Flocke by Mermet: Opaque, PVC-free fiberglass textile with acrylic backing. 17.1 oz/sq yd, .024 inches thick. Fire rating: NFPA 701, both small- and large-scale tests. Antistatic treated. Draper shades made with this fabric are GreenSpec listed.

      - Obion by Mermet: Translucent pvc free,  fiberglass textile with white exterior and soft woven interior. 10.03 oz/sq yd., .018 inches thick. Fire rating: NFPA 701, both small- and large-scale tests, TM#1/California U.S. Title 19.

      - Roc-Rol by Rockland: PVC-free polyester-cotton blend, opaque. 13.5 oz/sq yd., .025 inches thick. Fire rating: NFPA 701. All colors are white on back side. U.S. Patent 5,741,582, Rockland Industries, Inc. Draper shades made with this fabric are GreenSpec listed.

      - SheerWeave Series SW7000 by Phifer: PVC-free polyester with acrylic foamed backing. .018 inches thick, 10 oz/sq yd, opaque. Draper shades made with this fabric are GreenSpec listed.

      - SheerWeave Series SW7100 by Phifer: PVC-coated Fiberglass laminated with a 2-ply PVC film. Fire rating: NFPA 701.023 inches thick, 19.8 oz/sq yd, opaque.

** NOTE TO SPECIFIER ** Refer to Draper product literature for available shade fabric colors and patterns. Select fabric colors and patterns from the following, delete those not applicable.

    - Color and pattern:  __________, No. _____.

    - Color and pattern: As indicated in Color Schedule on Drawings.

    - Color and pattern: As selected by Architect from manufacturer's standard range.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

** NOTE TO SPECIFIER **  Include one of the following paragraphs. Delete if not applicable.

    - Coordinate requirements for blocking and structural supports to ensure adequate means for installation of window shades.

    - Coordinate requirements for blocking, construction of shade pockets, and structural supports to ensure adequate means for installation of window shades.

** NOTE TO SPECIFIER **  Include one or more of the following paragraphs for window shades with headboxes recessed in ceilings. Delete those not applicable.

    - Coordinate installation of recessed shade pockets with construction of suspended acoustical panel ceilings specified in Section 09 51 23.

    - Coordinate installation of recessed shade pockets with construction of suspended gypsum board ceilings specified in Section 09 21 16.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Install roller shades level, plumb, square, and true.  Allow proper clearances for window operation hardware.

** NOTE TO SPECIFIER **  Draper Type D FlexShade Pockets are designed to be installed prior to or as part of ceiling system installation. Shade and operating mechanism can be site installed in pockets after construction operations that might damage shade are complete. If pockets are to be installed. Include the following paragraph as applicable and delete if not applicable.

    - Shade pockets:

      - Install shade pockets prior to installation of suspended ceiling system. Attach to supporting structure with screws through top of pocket at 24 inches (610 mm) minimum centers.

      - Install shade pockets in conjunction with installation of suspended ceiling system. Attach to supporting structure with screws through top of pocket at 24 inches (610 mm) minimum centers.

      - Install corner pieces securely and in alignment with pockets.

      - Install pocket ends securely and in alignment with pockets.

      - After interior construction is essentially complete, install shade and operating mechanism in pocket.

** NOTE TO SPECIFIER ** Include the following paragraph if fascia or separate closure for site constructed recess is being provided. Note fascia is not applicable if pockets are being provided. Delete if not applicable.

    - Install the following items to conceal roller and operating mechanism. Do not use exposed fasteners.

** NOTE TO SPECIFIER ** Select items to conceal roller and operating mechanism from the following as applicable. Delete if not applicable.

      - Fascias.

      - Closure panels.

      - Endcaps.

  - TESTING AND DEMONSTRATION

    - Demonstrate operation of shades to Owner's designated representatives.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

  - SCHEDULES

** NOTE TO SPECIFIER ** Retain Paragraph below to suit project requirements. Identify products by name on the Drawings or alternately  use this paragraph to define the location of each type of material to be used. Delete if not required.

    - Refer to Drawings for shade types and locations.

END OF SECTION