Project Name:

SECTION 12492

Pleated Shades

# Part 1 - General

# DESCRIPTION

- Pleated Shades

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Submit working hand sample or mock up shade as required.

- Submit two 6" samples of material.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions LLC or architect approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - PLEATED SHADES

- Product: Graber® FashionPleat® Shades

- Headrail shall be 0.665" high x 1.540" deep extruded aluminum with a wall thickness of 0.045” and painted to coordinate with fabric color.

- Cord lock shall be a snap-in design of injection-molded thermoplastic incorporating a metal, free-floating, serrated cord-locking roller.

- Lift cord shall be .9 mm polyester. A snap tassel and joiner ball connect to a single Danskord for raising and lowering the shade. Cord, tassel and joiner ball are color coordinated with fabric.

- Pleated Fabric shall be selected from an assortment of fade resistant, 100% polyester fabrics. Fire retardant fabrics have met all NFPA-701 and California requirements for FR performance (Brighton only).

- Bottomrail shall be tubular shaped 0.024” thick phosphate-treated recycled steel and finished with a polyester primer and a top coat of polyester baked enamel. The bottomrail shall be finished with a 7/8” wide crowned PVC slat, which is fitted into the last fabric pleat.

Options:

Continuous Cord Loop Lift System: A continuous white cord loop controls lowering and raising shade (headrail measures 1 7/16” high by 2” deep).

Cordless Lift System: Provides a lift and lowering operation without the use of any cord controls (headrail measures 1 7/16” high by 2” deep).

  - Fabrication

- Shades shall be fabricated according to specifications and accurate to tolerance established by SWF engineering standards

# Part 3 – Execution

  - Inspection

- installer shall be responsible for inspection of jobsite, approval of mounting surfaces, verification of field measurements and installation conditions.  Installation shall commence when satisfactory conditions are met.

  - Installation

- Install blinds in accordance with manufacturer’s instructions including recommended support brackets and fasteners.

- Install blinds with adequate clearance to permit smooth operation of the blinds.  Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

- Maintain and clean blinds in accordance with manufacturer’s instructions.

### End of Section