Project Name:

SECTION 12492

Horizontal Louver Blinds

# Part 1 - General

# DESCRIPTION

- 2" Horizontal Faux Wood Blinds

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Submit working hand sample or mock up blind as required.

- Submit two 6" samples of slat indicating color and dimensions.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions LLC or architect approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - HORIZONTAL BLINDS

- Product: Graber® Lake Forest™ 2” Faux Wood Blinds

- Color Name:      Color Number:

- SureClose® Headrail shall be 1 5/8" high x 2 1/4" wide x .022” thick U-shaped steel with 1/8” light blocking lip on the bottom centerline. The steel finishing process includes phosphate treatment for corrosion resistance, a chrome-free sealer, a low HAP urethane primer and a topcoat with low HAP polyester baked enamel.

- Cord tilter shall be a snap-in component incorporating a worm and pulley of low-friction thermoplastic and gear of nylon.  Tilt cords shall be secured to pulley and treated with tassels at tilt end.

- Cord lock shall be metal of a snap-in design incorporating a floating, shaft-type locking pin.  Cord lock shall incorporate a crash proof safety feature that will lock blind automatically upon release of cord.  End of lift cords will be treated with tassels.

- Braided ladder shall be made of 100% polyester incorporating two extra strength rungs per ladder to support slats. Standard ladder spacing is 44mm.

Options:

Fabric Tape (Lake Forest Premium)

Color Name:		Color Number:

- Slats shall be rigid UV stabilized flat PVC nominally 2” wide x .10” thick with straight cut edges. Slats meet the requirement of NFPA 701 Test Method 1-2004 edition.

Options:

2-1/2” Slat Width

NoHoles™ Slats

- Bottomrail shall be extruded foam PVC 9/16” high x 2” wide and shall coordinate with slats.

  - Fabrication

- Blinds shall be fabricated according to specifications and accurate to tolerance established by SWF engineering standards

# Part 3 – Execution

  - Inspection

- installer shall be responsible for inspection of jobsite, approval of mounting surfaces, verification of field measurements and installation conditions.  Installation shall commence when satisfactory conditions are met.

  - Installation

- Install blinds in accordance with manufacturer’s instructions including recommended support brackets and fasteners.

- Install blinds with adequate clearance to permit smooth operation of the blinds.  Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

- Maintain and clean blinds in accordance with manufacturer’s instructions.

End of Section