Manual LightBloc FlexShades

** NOTE TO SPECIFIER ** Draper, Inc.; window shade products.
 .
 This section is based on the products of Draper, Inc., which is located at:
411 S. Pearl P. O. Box 425 
Spiceland, IN 47385-0425
Toll Free Tel: 800-238-7999
Tel: 765-987-7999
Fax: 866-637-5611
Email: drapercontract@draperinc.com
Web: www.draperinc.com

 Draper is a leader in the manufacture of window shades, projection screens, and gymnasium equipment. Founded in 1902, Draper maintains a tradition of quality and service. Draper products include FlexShade roller-operated and fixed window shades.

 Window shades reflect light and heat from the sun, reducing the amount of energy required to cool the building. Motorized window shades can automatically adjust to compensate for the changing shade requirements throughout the day. DRAPER's solar control shading systems can contribute to LEED points and credits. In addition, shades can reduce or eliminate uncomfortable glare for workers, while still allowing ambient sunlighting and view-through to the outside. The construction of window shades has become more friendly to the environment as well by including PVC-free fabrics, recyclable fabrics and lean manufacturing processes.

 Our spectrum of fabric colors and styles is unmatched anywhere. And we offer the broadest variety of operator styles in the industry.

 This section can be used for specifying Draper Manual  LightBloc FlexShades, an opaque fabric window shade system for complete blackout of window opening.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER **  Delete items below not required for project.

    - Opaque window shade system:  Manually operated, roll-up fabric opaque window shade system for complete blackout of window opening including side and bottom channels, headbox, opacity plates, manual operator, and mounting hardware.

  - RELATED SECTIONS

** NOTE TO SPECIFIER **  Delete any sections below not relevant to this project; add others as required.

    - Section 06 10 00 - Rough Carpentry.

    - Section 07 90 00 - Joint Protection.

    - Section 09 21 16 - Gypsum Board Shaft Wall Assemblies.

    - Section 09 51 23 - Acoustical Tile Ceilings.

    - Section 11 52 13 - Projection Screens.

    - Section 11 52 16 - Projectors.

  - REFERENCES

** NOTE TO SPECIFIER **  Delete references from the list below that are not actually required by the text of the edited section.

    - NFPA 701-99 - Fire Tests for Flame-Resistant Textiles and Films.

    - GREENGUARD Environmental Institute Children & Schools

    - US Green Building Council.

  - SUBMITTALS

    - Submit under provisions of Section 01 33 26 - Source Quality Control Reporting:

    - Product Data:  Manufacturer's data sheets on each product specified, including:

      - Preparation instructions and recommendations.

      - Installation and maintenance instructions.

      - Styles, material descriptions, dimensions of individual components, profiles, features, finishes and operating instructions.

      - Storage and handling requirements and recommendations.

      - Mounting details and installation methods.

** NOTE TO SPECIFIER **  Include the following paragraph for projects with complex requirements for window treatment.  Delete if not required.

    - Shop Drawings:  Plans, elevations, sections, product details, installation details, operational clearances, wiring diagrams and relationship to adjacent work.

** NOTE TO SPECIFIER **  Delete the following paragraphs if LEED is not applicable.

    - LEED Submittals:  Provide documentation of how the requirements of Credit will be met.

    - Window Treatment Schedule:  For all roller shades.  Use same room designations as indicated on the Drawings, field verified window dimensions, quantities, type of shade, controls, fabric, and color, and include opening sizes and key to typical mounting details.

** NOTE TO SPECIFIER **  Delete Selection Samples paragraph if colors and shade fabrics have already been selected.

    - Selection Samples:  For each finish product specified, two complete sets of shade cloth options and aluminum finish color samples representing manufacturer's full range of available colors and patterns.

    - Verification Samples:  For each finish product specified, two complete sets of shade components, unassembled, demonstrating compliance with specified requirements.  Shade fabric sample and aluminum finish sample as selected, representing actual product, color, and patterns.  Mark face of material to indicate interior faces.

    - Maintenance Data:  Methods for maintaining roller shades, precautions regarding cleaning materials and methods, instructions for operating hardware and controls.

    - Manufacturer's Certificates: Certify products meet or exceed specified requirements.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Obtain roller shades through one source from a single manufacturer with a minimum of twenty years experience in manufacturing products comparable to those specified in this section.

    - NFPA Flame-Test: Passes NFPA 701. Materials tested shall be identical to products proposed for use.

** NOTE TO SPECIFIER **  Include a mock-up if the project size and/or quality warrant taking such a precaution.  The following is one example of how a mock-up on a large project might be specified.  When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up:  Provide a mock-up of one of each type roller shade assembly specified for evaluation of mounting, appearance and accessories.

      - Locate mock-up in window(s) designated by Architect.

      - Do not proceed with remaining work until mock-up is accepted by Architect.

  - DELIVERY, STORAGE, AND HANDLING

    - Do not deliver window shades until building is enclosed and construction within spaces where shades will be installed is substantially complete.

    - Deliver products in manufacturer's original, unopened, undamaged containers with labels intact.

    - Label containers and shades according to Window Shade Schedule.

    - Store products in manufacturer's unopened packaging until ready for installation.

  - SEQUENCING

    - Ensure that locating templates and other information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

  - PROJECT CONDITIONS

    - Install roller shades after finish work and ambient temperature, humidity and ventilation conditions are maintained at levels recommended for project upon completion.

  - WARRANTY

    - Hardware and Shade Fabric:  Draper's standard twenty-five year limited warranty.

** NOTE TO SPECIFIER **  Include the following paragraph as applicable.

    - Spring Roller Shades:  Draper's standard ten year limited warranty.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Draper, Inc., which is located at: 411 S. Pearl P. O. Box 425 ; Spiceland, IN 47385-0425; Toll Free Tel: 800-238-7999; Tel: 765-987-7999; Fax: 866-637-5611; Email: drapercontract@draperinc.com; Web: owww.draperinc.com

** NOTE TO SPECIFIER **  Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions:  Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01 60 00.

  - OPAQUE WINDOW SHADE SYSTEM

** NOTE TO SPECIFIER ** Draper LightBloc FlexShades are provided in three operating styles: chain and clutch, crank, and spring roller. Refer to Draper product literature for selection guidelines. Select operation type from the following. Delete those not required.

    - Operation Type: Bead chain and clutch operated, vertical roll-up, fabric, opaque window shade system, complete with headbox, side and sill channels for total opacity; LightBloc FlexShade System as manufactured by Draper, Inc.

    - Operation Type: Crank operated, vertical roll-up, fabric, opaque window shade system, complete with headbox, side and sill channels for total opacity; LightBloc FlexShade System as manufactured by Draper, Inc.

    - Operation Type: Spring roller operated, vertical roll-up, fabric, opaque window shade system, complete with headbox, side and sill channels for total opacity; LightBloc FlexShade System as manufactured by Draper, Inc.

** NOTE TO SPECIFIER ** For Chain and Clutch Operated LightBloc FlexShade System include the following paragraph. Delete if not required

    - Operation: Bead chain and clutch operating mechanism allowing shade to stop when chain is released. Designed never to need adjustment or lubrication. Provide limit stops to prevent shade from being raised or lowered too far.

      - Clutch mechanism: Fabricated from high carbon steel and molded fiberglass reinforced polyester.

** NOTE TO SPECIFIER ** Select stainless steel or plastic bead chain from the following. Delete the one not required.

      - Bead chain loop: Stainless steel bead chain hanging at side of window.

      - Bead chain loop: Plastic bead chain hanging at side of window, Ivory, Grey, or Black color as selected by Architect.

      - Idler Assembly: Provide roller idler assembly of molded nylon with adjustable length idler pin to facilitate easy installation, and removal of shade for service.

** NOTE TO SPECIFIER **For Crank Operated LightBloc FlexShade System include the following paragraph. Delete if not required.

    - Operation: Crank and gear box operated shades.

      - Gear box: Die cast aluminum and steel construction. Injection molded nylon and steel crank insert installed in roller to engage gear box.

      - Crank handle:

** NOTE TO SPECIFIER **  Crank handle can be detachable or permanently mounted, select one of the following paragraphs.

        - Detachable anodized aluminum and chrome plated steel handle.

        - Permanently mounted anodized aluminum and chrome plated steel handle.

** NOTE TO SPECIFIER **  Crank handle can be provided in lengths up to 10 feet., select one of the following paragraphs.

        - Size: 4 feet (122 cm).

        - Size: 6 feet (183 cm).

        - Size: 10 feet (305 cm).

      - Idler assembly: Provide roller idler assembly of molded nylon with adjustable length idler pin to facilitate easy installation, and removal of shade for service.

** NOTE TO SPECIFIER **For Spring Roller Operated LightBloc FlexShade include the following paragraph. Delete if not required.

    - Operation: Metal roller containing heavy-duty spring with positive locking mechanism permitting shade to be stopped at each half turn of the roller. Spring sized by manufacturer to accommodate shade size. Provide pull and latching chain attached to bottom shade slat and catch attached to sill channel. Chain inserted in catch to keep roller under constant tension. Provide roller idler assembly of molded nylon with zinc-plated steel pin.

    - Roller Tube: Fabricated from extruded aluminum, galvanized steel, or enameled steel. Diameter, wall thickness, and material selected by manufacturer to accommodate shade type and size. Fabric connected to the roller tube with LSE (low surface energy) double sided adhesive specifically developed to attach coated textiles to metal. Adhesive attachment to eliminate horizontal impressions in fabric.

    - Headbox: Consists of extruded aluminum sections with endcaps and opacity plates.

      - Size: 4-1/8 inches (105 mm) high by 3-1/2 inches (89 mm) wide by length required for shade being provided.

** NOTE TO SPECIFIER **  Include the following paragraph for exposed headbox. Delete if not required.

      - L-shaped removable front face and bottom cover and L-shaped back and top.

** NOTE TO SPECIFIER ** Include the following paragraph for recessed headbox. Delete if not required.

      - U-shaped front, back, and top and removable bottom.

    - Endcaps: Stamped steel with universal design suitable for mounting to ceiling, wall, and jamb. Provide size compatible with roller size.

    - Side Channels: Double chamber fabricated from 0.06 inch (1.5 mm) thick extruded aluminum sections. One chamber accepts fabric and contains groove for fabric retainer. Other chamber accepts fabric guide and channel locator.

    - Sill channel: 0.06 inch (1.5 mm) thick extruded aluminum channel to receive slat bar and prevent light leakage.

    - Slat bar: Extruded aluminum bar attached to bottom of shade. Bar does not retract into headbox.

    - Channel locator: Injected molded nylon insert to align side and sill channels with headbox.

    - Fabric guide: Plated steel transition for fabric rolling into side channel.

** NOTE TO SPECIFIER ** LightBloc FlexShades with SB9000 and SB9100 fabric use fabric retainers, that includes a patented system which enables shade to withstand reasonable air pressure differentials. However, extreme air currents or physical interference cannot be resisted by fabric retention system and may cause physical damage. Refer to Draper product literature for selection guidelines.

    - Fabric retainer: System designed to prevent disengagement of fabric from side channels due to normal variations of air pressure caused by doors opening, HVAC systems, and temperature differences between room and window well. System consists of horizontal steel stays installed in shade, covered with fabric, and spaced at regular intervals. Grommets installed through stays are held within groove of side channel chamber.

    - Opacity plates: Steel plates with rubber O rings installed on end caps to eliminate light leakage.

    - Exposed aluminum finish:

** NOTE TO SPECIFIER **  Select the finish required from the following paragraphs and delete those not required.

      - Finish: Clear anodized aluminum.

      - Finish: Black.

      - Finish: White.

      - Finish: Ivory.

      - Finish: Bronze.

      - Custom powder coat as selected by the Architect.

  - FABRIC

** NOTE TO SPECIFIER ** Refer to Draper product literature for available shade fabrics and selection guidelines. Select fabric(s) and color as applicable from the following and delete those not applicable. Use the schedule at the end of this section if required to define location(s) of multiple fabrics.

    - Room Darkening Fabrics

      - SunBloc Series SB9000: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1. Washable and stain resistant. Wt. 12 oz/sq yd. Same color both sides, .015 inches thick.

      - SunBloc Series SB9100: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1. Washable and stain resistant. Wt. 12 oz/sq yd. White exterior in all colors, .013 inches thick.

      - CL8000 (Colony): Close woven fiberglass base textile with sun-resistant vinyl film securely bonded to each side. Fire rating: NFPA 701 Test 1996-1. Washable and stain resistant. 90 percent opaque. 9.0 oz/sq yd., .014 inches thick. Fabric is white on outside.

      - Flocke by Mermet: Opaque, PVC-free fiberglass textile with acrylic backing. 17.1 oz/sq yd, .024 inches thick. Fire rating: NFPA 701, both small- and large-scale tests. Antistatic treated. Draper shades made with this fabric are GreenSpec listed.

      - Obion by Mermet: Translucent pvc free,  fiberglass textile with white exterior and soft woven interior. 10.03 oz/sq yd., .018 inches thick. Fire rating: NFPA 701, both small- and large-scale tests, TM#1/California U.S. Title 19.

      - Roc-Rol by Rockland: PVC-free polyester-cotton blend, opaque. 13.5 oz/sq yd., .025 inches thick. Fire rating: NFPA 701. All colors are white on back side. U.S. Patent 5,741,582, Rockland Industries, Inc. Draper shades made with this fabric are GreenSpec listed.

      - SheerWeave Series SW7000 by Phifer: PVC-free polyester with acrylic foamed backing. .018 inches thick, 10 oz/sq yd, opaque. Draper shades made with this fabric are GreenSpec listed.

      - SheerWeave Series SW7100 by Phifer: PVC-coated Fiberglass laminated with a 2-ply PVC film. Fire rating: NFPA 701.023 inches thick, 19.8 oz/sq yd, opaque.

** NOTE TO SPECIFIER ** Refer to Draper product literature for available shade fabric colors and patterns. Select fabric colors and patterns from the following, delete those not applicable.

    - Color and pattern:  __________, No. _____.

    - Color and pattern: As indicated in Color Schedule on Drawings.

    - Color and pattern: As selected by Architect from manufacturer's standard range.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

** NOTE TO SPECIFIER **  Include one of the following paragraphs. Delete if not applicable.

    - Coordinate requirements for blocking and structural supports to ensure adequate means for installation of window shades.

    - Coordinate requirements for blocking, construction of shade pockets, and structural supports to ensure adequate means for installation of window shades.

** NOTE TO SPECIFIER **  Include one or more of the following paragraphs for window shades with headboxes recessed in ceilings. Delete those not applicable.

    - Coordinate installation of recessed shade pockets with construction of suspended acoustical panel ceilings specified in Section 09 51 23.

    - Coordinate installation of recessed shade pockets with construction of suspended gypsum board ceilings specified in Section 09 21 16.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Install roller shades level, plumb, square, and true.  Allow proper clearances for window operation hardware.

    - Install headbox, side channels, and sill channel with sealant specified in Section 07 90 00 - Joint Protection.

    - Position shades level, plumb, and at proper height relative to adjacent construction. Secure with fasteners recommended by manufacturer.

  - TESTING AND DEMONSTRATION

    - Test window shades to verify that operating mechanism, fabric retainer, and other operating components are functional. Correct deficiencies.

** NOTE TO SPECIFIER **  Select one type operating mechanism from the list below. Delete those not applicable.

      - Crank.

      - Chain and clutch.

      - Spring Roller.

    - During daylight hours, lower shades and turn off interior lights. Verify that there are no light leaks at perimeter or within shade assembly. Correct deficiencies.

    - Demonstrate operation of shades to Owner's designated representatives.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

  - SCHEDULES

** NOTE TO SPECIFIER ** Retain Paragraph below to suit project requirements. Identify products by name on the Drawings or alternately  use this paragraph to define the location of each type of material to be used. Delete if not required.

    - Refer to Drawings for shade types and locations.

END OF SECTION