Project Name:

SECTION 12492

Horizontal Louver Blinds

# Part 1 - General

# DESCRIPTION

- 1/2" Horizontal Louver Blinds With Aluminum Slats.

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Submit working hand sample or mock up blind as required.

- Submit two 6" samples of aluminum slat indicating color and dimensions.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions LLC or architect approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - HORIZONTAL BLINDS

- Product: Bali® Standard Micro Blinds

- Color Name:       Color Number:

- Headrail shall be 1" high x 1" wide x .025” thick U-shaped steel. The steel finishing process includes phosphate treatment for corrosion resistance, a chrome-free sealer, a low HAP urethane primer and a topcoat with low HAP polyester baked enamel.

- Tilter shall be made of injection-molded thermoplastics for smooth low-friction operation and will incorporate a clutch mechanism to prevent damage due to over tilting.

Options:

Tilt limiter will limit slat operation between a select range of operation. Select degrees up and degrees down from horizontal (up/down settings are nominally 0, 20, 40, 60 and 75):

Degrees up and       degrees down from horizontal

- Cord lock shall be metal of a snap-in design incorporating a floating, shaft-type locking pin and shall incorporate a crash proof safety feature that will lock blind automatically upon release of cord.

Options:

Top lock cord lock provides for only two blind positions, fully down or fully raised.

Ring pull provides a single plated steel ring in lieu of tassels with a nominal 4" cord length

- Braided ladder shall be made of 100% polyester yarn incorporating two extra strength rungs per ladder for slat support. Standard ladder spacing shall be 12.5mm.

- Slats shall be 5000 series cold-rolled aluminum containing the maximum allowable recycled content to produce a high strength and corrosion resistant flexible product. Slats shall be nominally 1/2” wide x .006" thick and processed with Advanced Finishing Technology (AFT), providing a smooth, hard, less porous surface.  AFT delivers anti-static performance to repel dust. Slats shall be treated with a topcoat of polyester baked enamel.

- Bottomrail shall be tubular shape made of phosphate-treated steel for corrosion resistance and finished with a chrome-free sealer, low HAP urethane primer and a topcoat of low HAP polyester baked enamel and shall measure .025" thick.

  - Fabrication

- Blinds shall be fabricated according to specifications and accurate to tolerance established by SWF engineering standards

# Part 3 – Execution

  - Inspection

- installer shall be responsible for inspection of jobsite, approval of mounting surfaces, verification of field measurements and installation conditions.  Installation shall commence when satisfactory conditions are met.

  - Installation

- Install blinds in accordance with manufacturer’s instructions including recommended support brackets and fasteners.

- Install blinds with adequate clearance to permit smooth operation of the blinds.  Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

- Maintain and clean blinds in accordance with manufacturer’s instructions.

### End of Section